# Optimized Flocking of Autonomous Drones in Confined Environments

Authors: Gabor Vasarhelyi, Csaba Viragh, Gergo Somorjai, Tamas Nepusz, A.E. Eiben, Tamas Vicsek

Published: 2018 — Science Robotics, Vol. 3, Issue 20, eaat3536

View Original Paper

# Overview

This groundbreaking paper demonstrated the first large-scale outdoor drone swarm operating without any central control. Thirty autonomous drones flew at 8 meters per second in confined environments, avoiding each other and obstacles using only local communication. The key innovation was using evolutionary optimization to tune the flocking parameters — rather than hand-tuning, they let artificial evolution discover the best settings.

# The Challenge

Real-world flocking is dramatically harder than simulation. Drones have GPS noise (meters of error), communication delays, wind gusts, motor variations, and limited sensing range. Hand-tuning dozens of parameters (how strongly to repel, how much to align, at what distance to react) for these messy real-world conditions is nearly impossible.

# The Solution: Evolutionary Optimization

## Step 1: Define the Flocking Model

Start with a physics-based flocking model with tunable parameters: repulsion strength, repulsion range, alignment weight, friction coefficient, preferred speed, communication range, etc. Each set of parameters produces different swarm behavior.

## Step 2: Define Fitness Metrics

Create measurable "order parameters" that quantify swarm quality: average alignment (are drones flying the same direction?), minimum distance (is anyone too close?), correlation (are they moving cohesively?), velocity match (are speeds similar?). Good parameters produce high fitness scores.

## Step 3: Evolutionary Search

Run thousands of simulations with different parameter combinations. The best-performing sets "survive" and combine to create new candidates (like biological evolution). Over many generations, the algorithm discovers parameter settings optimized for real-world conditions including noise and delays.

# Real-World Analogy: Perfecting a Recipe

Imagine you want the perfect chocolate chip cookie recipe. You could spend years hand-tuning each ingredient amount. Instead, you bake 1000 random variations, taste-test them all, keep the top 10 recipes, create new variations by combining their best features, and repeat. After many rounds, you converge on a recipe that is robustly delicious — even if the oven temperature varies or the flour is slightly different.

# Key Takeaways

- First demonstration of 30 drones flocking autonomously outdoors at high speed.

- Evolutionary optimization finds robust parameters that work despite real-world noise.

- Fitness functions based on measurable order parameters enable automated tuning.

- The approach bridges simulation and reality — parameters optimized in simulation transfer to real drones.

- Decentralized control scales: the same algorithm works for 10 or 100 drones.

© 2025 Sim Vattanac | vattanacsim99@gmail.com