Decentralized Multi-UAV Formation Control

Saska et al. — Multi-Robot Systems Lab, CTU Prague

Swarm Drone Research Paper #13 — Data Science Mega Document Research

# Chapter 1: Research Overview

Martin Saska’s Multi-Robot Systems (MRS) Lab at the Czech Technical University in Prague has produced one of the most comprehensive bodies of work on GPS-denied, infrastructure-free multi-UAV formation flight. Their publications span IEEE RA-L, ICRA, and IROS from 2017 to 2023.

The central achievement is a complete decentralized pipeline: from onboard UV-LED relative localization to formation control, validated in real outdoor experiments with up to 10 UAVs flying autonomously without GPS, motion capture, or any external positioning infrastructure.

- Lead Author: Martin Saska, Associate Professor, CTU Prague

- Group: Multi-Robot Systems Lab (mrs.felk.cvut.cz)

- Platform: Custom quadrotor MAVs with UV-LED markers

- Key Innovation: GPS-free formation flight using onboard visual relative localization

- Open Source: MRS UAV System (github.com/ctu-mrs)

# Chapter 2: Why GPS-Free Formation Matters

Most multi-drone demonstrations rely on GPS for outdoor flights or motion-capture systems for indoor experiments. Both have critical limitations that prevent real-world deployment in many important scenarios.

| Infrastructure | Limitation | Impact |
| --- | --- | --- |
| GPS | 2–5m accuracy; denied indoors, under canopy, in urban canyons | Cannot fly tight formations; fails in many operational environments |
| Motion Capture | Requires expensive external cameras; small volume only | Lab-only; cannot be deployed in the field |
| UWB Beacons | Requires pre-installed anchor nodes | Not available in unknown or remote environments |
| RTK GPS | Expensive ($1000+/drone); requires base station | Better accuracy but still infrastructure-dependent |

For real applications — search and rescue in collapsed buildings, forest monitoring under canopy, military operations in contested airspace, cave exploration — drones must localize relative to each other using only what they carry onboard. This is the fundamental problem Saska’s group addresses.

# Chapter 3: System Architecture

## Hardware Stack

| Component | Specification | Purpose |
| --- | --- | --- |
| Platform | Custom quadrotor MAVs (~500g, ~25cm) | Lightweight, agile, sufficient payload |
| Computer | Intel NUC or Raspberry Pi 4 | Runs ROS, image processing, control |
| Camera | Global-shutter USB camera + UV-pass filter | Detects UV-LEDs; rejects ambient light |
| Markers | UV-LED arrays with unique blink patterns | Inter-drone identification at 30+ Hz |
| IMU | 3-axis accelerometer + gyroscope | Attitude estimation between camera frames |
| Flight Controller | Pixhawk running PX4 | Low-level attitude and rate control |
| Communication | WiFi mesh (minimal bandwidth) | High-level coordination only |

## Software Pipeline

- UV camera captures image; UV-pass filter isolates LED markers from background.

- Blob detection finds LED centroids in the image as (u, v) pixel coordinates.

- Blink pattern matching identifies which neighbor each detected LED belongs to.

- Range estimation: d = (f × L) / p, where f = focal length, L = physical LED spacing, p = pixel spacing.

- Multi-target Kalman filter fuses camera detections with IMU predictions for smooth tracking.

- Formation controller computes velocity commands based on relative position errors.

- Pixhawk PX4 executes velocity commands via cascaded PID loops.

# Chapter 4: Onboard Relative Localization

The localization system is the core technical contribution. Each drone estimates the 3D relative position of its neighbors using only its onboard camera and the UV-LED markers on neighboring drones.

## UV-LED Detection

Each drone carries UV-LEDs that blink in a unique binary pattern (e.g., drone 1: 1010, drone 2: 1100). A UV-pass filter on the camera blocks visible light, making the LEDs appear as bright dots on a nearly black background. This makes detection trivially robust — a simple threshold operation extracts LED positions at 30+ fps even in bright sunlight.

## Range and Bearing Estimation

Range:   d = (f × L) / p

Bearing: θ = atan2(u - cx, f),  φ = atan2(v - cy, f)

where f is the focal length, L is the known physical LED spacing, p is the observed pixel spacing, and (cx, cy) is the camera principal point. This gives a full 3D relative position (range + azimuth + elevation) from a single monocular camera frame.

# Chapter 5: Formation Control Law

The decentralized formation controller drives each drone toward its desired position relative to its neighbors using a virtual-structure approach.

u_i = K_p × Σ_j (p_ij - p_ij*) + K_d × Σ_j (v_ij - v_ij*)

where p_ij is the measured relative position of neighbor j, p_ij* is the desired formation offset, and K_p, K_d are proportional and derivative gains.

## Formation Types

- Rigid formations: Line, V-shape, triangle, diamond — fixed inter-drone distances; the entire formation translates and rotates as a rigid body.

- Flexible formations: Scale, stretch, and deform on command — useful for adapting to obstacles, narrow passages, or changing mission requirements.

- Formation transitions: Smooth morphing between shapes (e.g., V → line → circle) during flight, managed by interpolating the desired offset vectors.

# Chapter 6: Consensus-Based Coordination

For the swarm to move as a coherent unit, drones must agree on a common reference velocity and heading. This is achieved through a distributed consensus protocol.

Each drone computes a weighted average of its own velocity estimate and those received from neighbors. The consensus converges exponentially fast on connected communication graphs.

## Leader-Follower Alternative

In leader-follower mode, one designated drone follows a mission waypoint plan while all others maintain their formation offsets relative to the leader. This is simpler to implement but introduces a single point of failure: if the leader fails, the entire formation is lost.

The consensus approach is more robust — any drone can be removed without catastrophic failure — but requires more communication bandwidth and introduces a small delay in velocity convergence.

# Chapter 7: Collision Avoidance

With drones flying 1–3m apart, collision avoidance is safety-critical. The system uses a reactive repulsive potential field that overrides the formation controller when drones get too close:

F_repulsive = K_rep × (1/d - 1/d_safe)² × n_ij    when d < d_safe

This layer operates independently and takes absolute priority over formation commands. The safety distance d_safe is set based on the operating environment:

| Scenario | Min Distance | Rationale |
| --- | --- | --- |
| Indoor | 0.8m | Low wind; prop wash dominant |
| Outdoor calm | 1.5m | Localization noise margin |
| Outdoor windy | 2.5m | Wind gusts cause position spikes |

# Chapter 8: Experimental Results

The CTU Prague group conducted extensive real outdoor flight tests that validate the complete system:

- Up to 10 UAVs flying in formation simultaneously, fully autonomous, with no GPS.

- Formation accuracy: ±0.3m position error at 2m inter-drone spacing.

- Localization rate: 30 Hz onboard processing using approximately 15% CPU on Raspberry Pi 4.

- Wind tolerance: Stable formation flight in winds up to 5 m/s (light breeze).

- Formation transitions: Smooth switching between V, line, diamond, and circle formations during flight.

- Longest continuous formation flight: approximately 8 minutes (limited by battery).

| Metric | GPS-Based Systems | Saska UV-LED System |
| --- | --- | --- |
| Position accuracy | 2–5m (standard GPS) | ±0.3m relative |
| Update rate | 5–10 Hz | 30+ Hz |
| Infrastructure needed | GPS satellites | None |
| Indoor capability | No | Yes |
| Jamming resistance | Vulnerable | Immune (optical) |

# Chapter 9: The MRS UAV System

The CTU Prague group has open-sourced their entire software stack as the MRS UAV System, available on GitHub. This has enabled other research groups worldwide to build on their work.

- Full ROS integration with modular architecture.

- Simulation environment (Gazebo) for development and testing before real flights.

- Support for multiple UAV platforms (custom builds, DJI, Crazyflie).

- State estimation, control, and planning modules that can be used independently.

- Active community with regular updates and documentation.

The open-source release has been adopted by over 30 research groups and has become one of the standard platforms for multi-UAV research.

# Chapter 10: Comparison with Other Approaches

| System | Localization | Scale | GPS-Free? |
| --- | --- | --- | --- |
| Saska (CTU Prague) | UV-LED + camera | 10 UAVs outdoor | Yes |
| Crazyswarm (Preiss) | Motion capture | 49 drones indoor | Indoor only |
| Intel Shooting Star | RTK GPS | 2,000+ drones | No (GPS required) |
| Vásárhelyi flocking | GPS + local comms | 30 drones outdoor | No |
| DJI FlightHub | RTK GPS + cloud | Commercial fleet | No |

Saska’s system uniquely provides GPS-free outdoor formation flight with no external infrastructure. The trade-off is smaller scale (10 vs. thousands) and line-of-sight requirements for the camera.

# Chapter 11: Applications

- Search and rescue: Coordinated UAV teams sweeping collapsed structures where GPS is blocked.

- Forest monitoring: Formation flight under canopy for fire detection, wildlife tracking, and canopy mapping.

- Infrastructure inspection: Coordinated inspection of bridges, power lines, and pipelines with systematic coverage.

- Military and defense: GPS-denied environments are standard in contested airspace; optical localization is unjammable.

- Cave and tunnel exploration: Multi-drone formations mapping underground spaces with no satellite access.

- Precision agriculture: Low-altitude coordinated spraying or sensing in orchards where GPS multipath degrades accuracy.

# Chapter 12: Limitations & Future Directions

- Line of sight: Camera must see neighbor LEDs; occlusion by obstacles or other drones breaks localization. Multi-camera setups or UWB fusion could mitigate this.

- Scale: Tested with up to 10 UAVs; larger swarms need hierarchical sub-group management.

- Battery life: ~8 min flight time limits mission duration. Better batteries or relay strategies needed.

- Weather: UV-LEDs work in sunlight but degrade in heavy rain, fog, or snow. Sensor fusion with radar or acoustic could help.

- 3D formations: Primarily tested at uniform altitude; full 3D formations with altitude diversity require stereo cameras or multi-frequency LEDs.

- Dynamic obstacles: Current avoidance is reactive only; predictive avoidance of moving objects is an open challenge.

# Chapter 13: Key Takeaways

- GPS is a single point of failure for outdoor drone operations. Onboard relative localization eliminates this dependency.

- UV-LED markers provide a simple, cheap, and robust visual localization solution that works in direct sunlight.

- Decentralized control eliminates coordinator bottlenecks and scales naturally with swarm size.

- ±0.3m accuracy at 30 Hz significantly outperforms GPS-based formation flying in both precision and update rate.

- Safety-first collision avoidance always takes priority over formation accuracy.

- Real outdoor validation with 10 UAVs demonstrates the system is field-ready, not just a simulation exercise.

- Open-source release (MRS UAV System) has catalyzed adoption by 30+ research groups worldwide.

Saska’s work represents one of the most complete and practically validated approaches to autonomous multi-UAV formation flight. The combination of GPS-free localization, decentralized control, and extensive outdoor testing makes this a cornerstone contribution to the field of swarm drone research.